Escaux UCS interoperability with IMS Mobistar

Document Control

| Version | Date | Notes |
| --- | --- | --- |
| 0.1 | 08/07/2014 | First draft |
| 1.0 | 11/07/2014 | Review by Dominique Bogaerts (Escaux) |
| 1.1 | 08/12/2014 | ESCAUX SIP Trunk settings Updated by Bogaerts (ESCAUX) |

Contact

All inquiries related to the feature described in this document can be emailed to support@netaxis.be.

This document will not be transmitted to customer integrally, the needed chapter have to be extracted and for the customer & other Mobistar parties than engineering the only technical Point of contact is Mobistar Network IMS Engineering

Acronyms

DDI : Direct Dial In number

IMS : IP Multimedia Subsystem

LAN : Local Area Network

SDP: Session Description Protocol

SIP: Session Initiation Protocol

UCS: Unified Communication Solution

References

|  | Title | Version |
| --- | --- | --- |
| 1 | Escaux Administration Guide: Unified Communication Solution 4.10. https://www.escaux.com/docs/SmpAdminGuide4d10.html | 2.2 |

Introduction

The purpose of this document is to provide configuration guidelines for connecting the Unified Communication Solution of Escaux (UCS) to the Mobistar IMS infrastructure.

The document does not deal with IP Telephony LAN design. More details are available in [1].

Definition:

Inbound call: call from IMS to UCS

Outbound call: call from UCS to IMS

IMS SIP Trunk: the SIP trunk between the UCS and the IMS

UCS Environment

The purpose of this section is to quickly describe the architecture of Escaux UCS. The UCS is made of Service Operational Points (SOPs) and a Service Management Point (SMP).

The realtime tasks are executed by SOPs. These are appliances that can be installed on-site for connectivity in order to be close to the users. The SOPs can also be installed remotely, as long as the phones and applications can reach them over a network.

Non-realtime tasks are split off in a centralized web interface: the SMP.

Software release

The software release of the UCS is 4.10

Solution Design

This chapter defines the different view of the Solution Design

IMS & ACCESS Infrastructure

Figure : IMS and access architecture

The Mobistar IMS Core & access network is composed with composed with different domains

The Access network enable to deliver a IPVPN over multi access technology (Fiber, DSL,…)

Figure : Access architecture

Solution for business continuity

Various solution could be minded, most typical are

- IP access redundancy (IP Access Redundancy)

- IP access redundancy + Escaux UCS redundancy

- Redundancy via VoIP enterprise Trunking

- ISDN GW backup into Escaux UCS

Please contact the engineering to review specific customer need for business continuity

Figure : Solution for business continuity

Provisioning parameters transmitted to the customer

| Parameter | Value example | Format | Definition |
| --- | --- | --- | --- |
| [CORP_SITE_PBX_NDG] | +3224312490 | Tel num (8 digits) | PBX NDG or tel number of pilot number PBX (1 head number) |
| [CORP_ID]-[CORP_SITE_ID]-NDG@sip.mobistar.be | 040000940-040000941-NDG@sip.mobistar.be | User name | Compilation of FFCLI &FCLI |
| [CORP_SITE_PBX_NDS_n] | +3289550940> +3289550947 | Tel num (8 digits) | PBX NDS ( tel number of DDI or NDS users behind PBX not including NDG) |
| [CORP_SITE_PBX_SIPPWD] | IMSHUAWEI | Random, with following format | SIP password for PBX |
| [VOIPT_CPETYPE_PUBSIG] | 212.224.148.97 | Public IP address | 1 Pub IP address per CPE Type per corporate - Client Side IP (this one is the one configured by customer in IPPBX). |
| [VOIPT_DOMAINNAME] | sip.mobistar.be | String | Host name part of SIP URI, SIP domain |
| [CORP_BACKUP] | +3289550000 | 9 digit tel number ex. 493262426 | Voice backup destination (national / international) mobile destination |
| [CORP_SITE_SUBNET | 192.168.50.1 | Private IP address (subnet) | Site (or corporate) subnet (main and remote) |
| [CORP_SITE_SUBNETMASK] | 255.255.255.0 | Private IP address (subnet mask) | Site (or corporate) subnet (main and remote) |

Table : configuration parameters provided by Mobistar

Supported Services

The purpose of this section is to list the services offered by Escaux UCS that have been validated with the SIP trunk of Mobistar:

- SIP Registration

- Basic incoming and outgoing calls

- Call hold

- Call Forwarding Unconditional

- Call Forwarding on No Answer

- Call forwarding to UCS

- Blind Transfer

- Consultative Transfer

- Conference

- Call Waiting

- Calling Line Identity Presentation

- Calling Line Identity Restriction

- DTMF

- Emergency calls

- Fax

- …

Configuration

The purpose of this section is to describe the configuration of the Escaux UCS, based on

- The parameters transmitted by Mobistar to the customer, as provided in section 3

- Other parameters described here.

Figure : configuration parameters inside the Enterprise

| Parameter | Value example | Format | Definition |
| --- | --- | --- | --- |
| [PHONE_SUBNET] | 10.0.12.0 | IP address | Subnet of the phones |
| [PHONE_SUBNETMASK] | 255.255.255.0 | Subnet mask | Subnet mask of the subnet [PHONE_SUBNET] |
| [UCS_IP_ADDR] | 10.0.12.11 | IP address | IP address of the UCS (access side) |
| [MOB_SUBNET] | 192.168.50.1 | IP address | Subnet of the UCS (Mobistar side) |
| [MOB_SUBNETMASK] | 255.255.255.0 | Subnet mask | Subnet mask of the subnet [MOB_SUBNET] |
| [UCS_MOB_IP_ADDR] | 192.168.50.12 | IP address | IP address of the UCS (Mobistar side) |
| [MGMT_SUBNET] | 10.0.12.0 | IP address | Subnet of the UCS (Management network) |
| [MGMT_SUBNETMASK] | 255.255.255.0 | Subnet mask | Subnet mask of the subnet [MGMT_SUBNET] |
| [UCS_MGMT_IP_ADDR] | 10.0.12.11 | IP address | IP address of the UCS (Management network) |
| [CPE_CORP_SITE_IP_ADDR] | 192.168.50.1 | IP address | IP address of Mobistar CPE |

Table : configuration parameters inside the Enterprise

Escaux UCS configuration

We only cover in this document the configuration objects related to the interconnection with the SIP trunk of Mobistar. For detailed information on how to configure the UCS, check [1].

Resources

The purpose of this section is to describe two resources configuration objects. The resources are the standard building blocks needed to set up the Escaux UCS solutions. UCS offers a wide variety of resources for a variety of purposes. The configuration objects “IP phone” and “Interface” are example of resources.

Interface

In SMP GUI: Resources -> Interface

The SIP trunk of Mobistar is represented inside the UCS as an Interface of type “SIP Trunk (OutgoingSIPTrunk)”.

Important parameters for configuring this interface are

| SIP Interface parameters |  |
| --- | --- |
| Parameters | Values |
| Gateway | [VOIPT_CPETYPE_PUBSIG] |
| Accout Data: User | [CORP_ID]-[ CORP_SITE_ID]-NDG@sip.mobistar.be |
| Password | [CORP_SITE_PBX_SIPPWD] |
| DTMF mode | RFC2833 |
| Register | Yes |
| User register | <phonenumber>@sip.mobistar.be |
| Auth register | [CORP_ID]-[ CORP_SITE_ID]-NDG@sip.mobistar.be |

Table : SIP Interface parameters for the Mobistar SIP trunk interface

| Advanced parameters |  |
| --- | --- |
| Parameters | Values |
| From domain | sip.mobistar.be |
| Restriction Group (Context) | FromOutside (see section 5.1.2 for more information) |
| CallerID Method | Use P-Preferred-id header NO CLIR (outgoing only) |
| Can reinvite | NO |
| User equals phone | YES |
| Allow T.38 | NO |

Table : Advanced parameters for the Mobistar SIP trunk interface

The following figure displays the Mobistar SIP trunk is defined with the ID “SOA10003”.

Figure : Escaux UCS Interface for Mobistar SIP Trunk

IP Phones

In SMP GUI: Resources -> IP Phones

The IP phones and faxes are represented inside the UCS as “IP phones” configuration objects. They are configured with their MAC address, types and versions. This configuration object also specifies the Restriction Group (see section 5.1.2 for more information on Restriction Groups).

The figure below shows an example of Polycom IP phone with MAC address 00:04:F2:AB:34:46.

Figure : IP phone configuration for a Polycom

The figure below shows an example of a Cisco SPA112, used to connect an analog fax device.

Figure : IP phone configuration for a Cisco SPA112

Site definition

In SMP GUI: Advanced -> Site Configuration

Sites containing IP phones can be configured inside the UCS. This is useful for Call Admission Control and Emergency call handing in case the network is composed of several sites interconnected by VPN links.

In a UCS connected with Mobistar, one site configuration object needs to be defined per postal code. This configuration object is used to add the prefix 969<postal code> to the dialed number for an emergency call, based on the site of origin of the call.

Here is an example of site definition for Ixelles/Elsene, with postal code 1050.

Figure : Site definition example

Network definition

In SMP GUI: Advanced -> Network Configuration

A site is composed out of one or various Local Area Networks (LAN). These LAN segments identify a particular network region. Local network(s) can be linked to site locations defined in section 5.1.2

| Network definition |  |
| --- | --- |
| Parameters | Values |
| Site | as defined in section 5.1.2 |
| Network address | [PHONE_SUBNET] |
| Netmask | [PHONE_SUBNETMASK] |

Table : Network Definition

Extensions

In SMP GUI: Directory -> Internal Directory

The purpose of this section is to briefly describe the concept of an extension in Escaux UCS. An extension is an internal phone number (user extension), but it can also be the trigger point for a call flow

User extension

Each user extension is included inside the Directory, including First Name, Last Name, Primary Phone (IP phones defined in section 5.1.1) and Call flow.

Figure : Extensions and Directory

One can modify each extension supplementary services by clicking on the icon  in front of the call flow name.

Figure : Profile Parameter Management (user extension)

Call Forwarding Unconditional can be enabled (“Forward all calls” should be set to “yes”).

The section “My communication flow” can be used to enable call waiting (“When I am on the phone, warn me for new incoming calls”), Forwarding busy (“When I am on the phone, forward my calls”). Call Forwarding on no reply is configured as a “Second action” (using, for example, “When I do not answer, send to my alternative number”, where the alternative number is defined in “General Settings”).

Extension for outgoing calls

Other extensions are not linked to specific phones or users. Their purpose is internal. In the context of SIP trunking, following extensions are important:

- The extension 89991001 is used for outgoing calls (with the call flow Template-OutgoingCallcatcher.Service)

- The extension 89991000 is used for incoming calls (with the call flow UC-Technical-IncomingCallcatcher.Service)

We can modify the properties of the call flow for outgoing calls by clicking on the icon  in front of the call flow profile name Template-OutgoingCallcatcher.Service.

This is where we can specify the ID of the SIP trunk that needs to be used for outgoing call (in our example on Figure 5, the SIP Trunk ID is “SOA10001).

This is also where we can set the emergency number configuration mode to “prefixed”: which is needed to add the prefix 969<postal code> to emergency numbers.

Figure : Profile Parameter Management (outgoingCallcatcher)

Note: as specified in Callflow Studio -> Callflow Assignment, the Callflow profile “Template-OutgoingCallcatcher.Service » is mapped to the Call flow root *136. The call flow with the root *136 is shown in Callflow Studio -> Callflow and contains the call handling logic for outgoing calls to Mobistar SIP trunking (including emergency call handling).

Extension for outgoing calls

We can modify the properties of the call flow for incoming calls by clicking on the icon  in front of the call flow name UC-Technical-IncomingCallcatcher.Service.

The parameter “Translate callers” should be set to “Yes”.

Figure : Profile Parameter Management (UC-Technical-IncomingCallcatcher)

Note: as specified in Callflow Studio -> Callflow Assignment, the Callflow profile “UC-Technical-IncomingCallcatcher.Service » is mapped to the Call flow root *126. The call flow with the root *126 is shown in Callflow Studio -> Callflow and contains the call handling logic for incoming calls from Mobistar SIP trunking.

Call Routing

Route Group

In SMP GUI: Call Routing -> Route Groups and Restriction Groups

A route group is simply a number of routes grouped together for a certain reason.

In the Figure below, several Route Groups are configured for breakout : mobile, national, international, premium, emergency, DefaultOut. One Route Group is configured for incoming calls :  catchAll

Figure : Route Groups

Restriction Group

In SMP GUI: Call Routing -> Restriction Groups Configuration

A restriction group is a list of route groups that will be used as permissions:

- All users in a restriction group can only call numbers that correspond to routes in the route groups of the restriction group.

- Incoming calls using a SIP Trunk in a Restriction Group can only call numbers that correspond to routes in the route groups of the restriction group.

In the configuration example of Figure 14,

- The Restriction Group FromOutside (specified at SIP Trunk group level, see section Figure 5) allows all routes in the route group catchall

- The Restriction Groups NoRestrict (specified at the level of the IP Phone see Figure 6) allows all routes in the route group default, incoming, internal, international, mobile, premium, national, emergency and service.

Figure : Restriction Groups

Note: the Precedence field allows to define a priority for each route group in the Restriction Group. This is useful when there are at least two routes that overlap to ensure that the right route will be selected. For two matching routes in distinct route groups, setting a smaller precedence to a route group ensures that the route of this group will be used. Therefore, the SOP will not necessarily select the route based on the longest matching prefix.

Route

In SMP GUI: Call Routing -> Route

A route maps a phone number pattern (for example, all numbers starting with 0049*) to an action.

To make patterns, following special characters can be used:

- X matches any digit from 0-9

- Z matches any digit from 1-9

- N matches any digit from 2-9

- [1237-9] matches any digit or letter in the brackets (in this example, 1,2,3,7,8,9)

- . wildcard, matches one or more characters

- ! wildcard, matches zero or more characters

In the example used in this document, a route can be part of one of the route group shown in Figure 13 (mobile, emergency, …). For breakout, the Action should be set to Goto.DefaultOut.1.1 in order to eventually reach the OutgoingCallCatcher service callflow described in section 5.1.4)

Figure : Routes

Incoming number mapping

In SMP GUI: Call Routing -> Incoming number mapping

Mapping incoming calls comes down to routing the incoming DDI numbers that you received from your phone operator to internal extensions. This can be done using an action, as shown in the list of the incoming number mapping:

Figure : External Number Mapping

Mapping these numbers onto an internal extension, you can trigger a phone (or a root extension (call flow) for automatic answering of calls). The following screenshot shows the detailed view when modifying or adding a mapping:

Figure : External Number Mapping details

Outgoing number mapping

In SMP GUI: Call Routing -> Outgoing number mapping

When dialing out, you can assign an external number that will be showed to the external contact, or you can choose to hide your number. There are five possibilities that you can select in the SMP:

- Existing: reuse the one that is currently configured.

- Hide: don't show any number when dialling out.

- Incoming: use the same number as the one your contacts use to call you directly (DDI).

- Default: use a default number, e.g. the main reception number.

- Other: enter any other number. Of course, limitations apply: you can only use numbers that your operator allows you to use.

ANNEX 1 Call Flows

Registration

Figure : Registration Call Flow

Basic outgoing call

Figure : Outgoing Call Flow

Basic incoming call

Figure : Incoming Call Flow

Blind call transfer

This call flow describes a scenario of a PSTN user A calling a user B behind the UCS. User B transfers blindly to PSTN user C. Note that the Media is anchored by the UCS after the transfer.

Figure : Blind Transfer Call Flow